# AS4113 Section 5.2.1.2 Test Case Verification Procedures

| Test ID | Objective | Message Type | Verification Points |
| --- | --- | --- | --- |
| RT-001 | RT accepts and stores BC-to-RT data word | BC to RT | RT correctly receives 1 data word and stores it in internal buffer |
| RT-002 | RT accepts multiple data words | BC to RT | RT receives multiple data words, respects word count, and stores them sequentially |
| RT-003 | RT transmits 1 data word to BC | RT to BC | RT sends correct data word in response to BC command |
| RT-004 | RT transmits multiple data words | RT to BC | RT sends all requested data words correctly and in proper timing |
| RT-005 | RT processes valid mode code (no data) | Mode Code | RT responds with status word; performs internal action (e.g., reset) |
| RT-006 | RT processes valid mode code (with 1 data word) | Mode Code | RT receives or transmits data and responds with status word |
| RT-007 | RT ignores message addressed to another RT | Address Filtering | RT remains silent when not addressed |
| RT-008 | RT handles broadcast message correctly | Broadcast | RT accepts data without responding as per MIL-STD-1553 rules |
| RT-009 | RT checks command parity | Error Handling | RT detects parity error in command word and does not respond |
| RT-010 | RT checks word count overflow | Error Handling | RT detects excess data word transmission and flags error (if implemented) |