# AS4113 Section 5.2.1.1 Test Case Verification Procedures

| Test ID | Objective | Message Type | Verification Points |
| --- | --- | --- | --- |
| BC-001 | Verify BC can request 1 data word from RT | RT to BC | BC sends command; RT returns 1 data word + status; BC decodes and validates |
| BC-002 | Verify BC can request multiple data words from RT | RT to BC | Correct data words received in sequence; word count and timing validated |
| BC-003 | Verify BC can transmit 1 data word to RT | BC to RT | BC sends command + 1 data word; RT receives; no response expected |
| BC-004 | Verify BC can transmit multiple data words to RT | BC to RT | BC sends N data words; RT acknowledges via status; no data returned |
| BC-005 | Verify BC can conduct RT-to-RT transfer | RT to RT | BC mediates: one RT transmits, one RT receives; both status words received |
| BC-006 | Send mode code without data | Mode Code | RT receives mode code; responds with correct status |
| BC-007 | Send mode code with one data word | Mode Code | RT sends/receives single word depending on mode code; status word returned |
| BC-008 | Broadcast transmission of data | Broadcast | All RTs accept data silently; no responses expected |
| BC-009 | Broadcast mode code | Broadcast | RTs update internal state; no response; verify no unexpected transmission |
| BC-010 | Send command with word count 0 | Edge Case | BC/RT handle 0-word command appropriately (mode code or reserved behavior) |
| BC-011 | Send command with maximum word count (32) | Edge Case | BC sends 32 data words; receiver handles and validates all words |
| BC-012 | Send command to non-existent RT | Error Handling | No response; BC detects timeout or marks error |
| BC-013 | No response from RT | Error Handling | BC detects timeout and handles according to protocol |
| BC-014 | RT responds too early | Timing Error | BC detects early response and flags timing violation |
| BC-015 | RT responds too late | Timing Error | BC detects delayed response and flags timing violation |
| BC-016 | RT responds with incorrect status | Error Handling | BC detects mismatch in expected status word bits |
| BC-017 | RT responds with parity error | Error Handling | BC detects parity fault in response word |
| BC-018 | RT sends unexpected word count | Error Handling | BC detects more/fewer data words than commanded |
| BC-019 | Verify inter-word and inter-message gap timing | Protocol Timing | All gaps and spacing validated against 1553 timing rules |
| BC-020 | Sync pattern is corrupted mid-transfer | Fault Injection | BC fails to decode word and flags sync error |