| WORK PACKAGE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| WORK PACKAGE # |  |  | DATE |  | CLASSIFICATION |
| DOUBLE CLICK |  |  | 12 April 2022 |  | RESTRICTED |
| TITLE/SUBJECT: |  |  |  |  |  |
| MIL-STD-1553 Dual redundant bus Remote Terminal VHDL core |  |  |  |  |  |
| DISTRIBUTION: |  |  |  |  |  |
| Master: EBD SAP, ----- |  |  |  |  |  |
| CONCLUSIONS/DECISIONS/AMENDMENTS: |  |  |  |  |  |
| This work package describes the FPGA firmware implementation for the MIL-STD-1553 dual redundant bus remote terminal (RT) core implementation. The core is designed and optimized for the Intel (Altera) range of products, however, with simple changes alternative architectures to the memory blocks and PLL components, the implementation can be ported to other vendors. |  |  |  |  |  |
| AUTHOR: I Kruger |  | SIGNATURE |  |  |  |
| APPROVED BY: PER Smit |  | SIGNATURE |  |  |  |
| PREVIOUS WP #: | N/A | SUPERSEDING WP #: |  | N/A |  |
| DATE: | N/A | DATE: |  | N/A |  |

CONTENTS

Page

ABBREVIATIONS	4

1.	Introduction	5

2.	REFERENCE DOCUMENTS	5

2.1	APPLICABLE DOCUMENTS	5

3.	Concept of operations	5

4.	Memory MAP	6

4.1	High level memory map	6

4.2	Registers	6

4.2.1	reg_status (0x0000)	8

4.2.2	reg_intr_mask (0x0001)	9

4.2.3	reg_clear_bits (0x0002)	9

4.2.4	reg_tmr_1us (0x0003)	10

4.2.5	reg_tmr_GAP (0x0004)	10

4.2.6	reg_tmr_NRP (0x0005)	11

4.2.7	reg_tmr_REP (0x0006)	11

4.2.8	reg_tmr_SAF (0x0007)	11

4.2.9	reg_legal_subaddr1 (0x0008)	12

4.2.10	reg_legal_subaddr2 (0x0009)	12

4.2.11	reg_legal_rx_mode1 (0x000A)	12

4.2.12	reg_legal_rx_mode2 (0x000B)	12

4.2.13	reg_legal_tx_mode1 (0x000C)	13

4.2.14	reg_legal_tx_mode2 (0x000D)	14

4.2.15	reg_legal_brdcst_mode1 (0x000E)	14

4.2.16	reg_legal_brdcst_mode2 (0x000F)	14

4.2.17	reg_status_rxed1 (0x0010) and reg_status_rxed2 (0x0020)	15

4.2.18	reg_mode_cmd_rxed1 (0x0011) and reg_mode_cmd_rxed2 (0x0021)	16

4.2.19	reg_mode_data_rxed1 (0x0012) and reg_mode_data_rxed2 (0x0022)	16

4.2.20	reg_rx_cmd1 (0x0013) and reg_rx_cmd2 (0x0023)	16

4.2.21	reg_bus1_status (0x001E) and reg_bus2_status (0x002E)	17

4.2.22	reg_bus1_mask (0x001F) and reg_bus2_mask (0x002F)	17

4.2.23	Timestamp (0x0030 – 0x0033)	18

4.2.24	reg_rt_control (0x0034)	18

4.2.25	reg_gID (0x0036)	19

4.2.26	reg_fw_version (0x0037)	19

4.2.27	reg_rt_addr (0x0038)	19

4.3	Transmit RAM	19

4.4	Receive RAM	20

5.	Remote Terminal Configuration	21

FIGURES

Figure 1: Top level architecture	5

tables

Table 1: High Level Memory Map	6

Table 2: Registers	8

Table 3: Transmit RAM Memory Map	20

Table 4: Receive RAM Memory Map	21

APPENDICES

No table of figures entries found.

# ABBREVIATIONS

| ABBREVIATION | DESCRIPTION |
| --- | --- |
| FPGA | Field Programmable Gate Array |
| ADC | Analog to Digital Converter |

# Introduction

MIL-STD-1553 is a standard defining a redundant bus topology for use in military equipment. MIL-STD-1553 forms the basis of the communication bus for the MIL-STD-1760 specification.

This core implementation is focussed specifically on the implementation of a dual redundant remote terminal device, implemented on Intel architecture devices.

The top-level architecture of the implementation is shown in Figure 1. The design makes use of two identical remote terminal media access controllers (MAC) and a common CPU and RAM interface. The MAC of each bus connects to the physical layer device (PHY) located on the PCB external to the FPGA.

Figure 1: Top level architecture

# REFERENCE DOCUMENTS

## APPLICABLE DOCUMENTS

MIL-STD-1553C

MIL-HDBK-1553A

MIL-STD-1760E

MIL-HDBK-1760A

# Concept of operations

The MIL-STD-1553 Standard (hereafter referred to as “Standard”) requires a remote terminal to respond within 150ms of power on to communications from the bus controller. The initial reply may utilize the BusBusy bit to indicate that the RT is not yet ready, however, within 500ms the RT needs to clear the BusBusy bit and reply with the correct message replies as defined in the Standard. Typically, in the application of MIL-STD-1760, the RT shall reply with the correct store identification message. The MAC design is such that it would immediately after FPGA configuration completion be able to start communicating with the host. The maximum internal configuration time for the MAX10 10M16 device using an encrypted compressed image is 22.3ms. This allows the design to meet the 150ms requirement. In order for the design to meet the 500ms requirement, the host processor software needs to define the reply messages by configuring the Transmit RAM location for each message and clear the BusBusy bit in the RT control register (reg_rt_control). During this initial power on configuration, the host may also configure additional settings for the RT core such as enabling interrupts for received message and error conditions.

The CPU and RAM interface block contains the registers to control the core as well as transmit and receive RAM.

# Memory MAP

## High level memory map

The memory map as indicated in Table 1 shows the main sections of the memory as mapped via the CPU interface. These address areas and values need to be added to the CPU memory offset defined during the instantiation of the core in the host CPU system.

The details of the first four sections for the “Registers” are given in section 4.2 and the transmit and receive RAM in section 4.3 and in section 4.4 respectively.

| Start | End | Size(words) | Function |
| --- | --- | --- | --- |
| 0x0000 | 0x000F | 16 | Common registers |
| 0x0010 | 0x001F | 16 | Bus1 Registers |
| 0x0020 | 0x002F | 16 | Bus2 Registers |
| 0x0030 | 0x003F | 16 | Common registers |
| 0x0040 | 0x03FF | 960 | Reserved |
| 0x0400 | 0x07FF | 1024 | Transmit RAM |
| 0x0800 | 0x0BFF | 1024 | Receive RAM |

Table 1: High Level Memory Map

## Registers

The remote terminal core is configured and controlled via the host CPU using a memory mapped interface. The blue coloured registers are general for both busses, the green is bus 1 specific register and the orange are bus2 specific register.

This section describes the CPU interface and registers.

| Registers | Address | Read/Write | Default |
| --- | --- | --- | --- |
| reg_status | 0x0000 | Read | 0x0000 |
| reg_intr_mask | 0x0001 | Write | 0x0000 |
| reg_clear_bits | 0x0002 | Read/Write | 0x0000 |
| reg_tmr_1us | 0x0003 | Read/Write | 0x0063 |
| reg_tmr_GAP | 0x0004 | Read/Write | 0x018F |
| reg_tmr_NRP | 0x0005 | Read/Write | 0x0577 |
| reg_tmr_REP | 0x0006 | Read/Write | 0x018F |
| reg_tmr_SAF | 0x0007 | Read/Write | 0x031F |
| reg_legal_subaddr1 | 0x0008 | Read/Write | 0xFFFF |
| reg_legal_subaddr2 | 0x0009 | Read/Write | 0xFFFF |
| reg_legal_rx_mode1 | 0x000A | Read/Write | 0x0000 |
| reg_legal_rx_mode2 | 0x000B | Read/Write | 0x0032 |
| reg_legal_tx_mode1 | 0x000C | Read/Write | 0x01FF |
| reg_legal_tx_mode2 | 0x000D | Read/Write | 0x000D |
| reg_legal_brdcst_mode1 | 0x000E | Read/Write | 0x01FA |
| reg_legal_brdcst_mode2 | 0x000F | Read/Write | 0x0032 |
| reg_status_rxed1 | 0x0010 | Read | 0x0000 |
| reg_mode_cmd_rxed1 | 0x0011 | Read | 0x0000 |
| reg_mode_data_rxed1 | 0x0012 | Read | 0x0000 |
| reg_rx_cmd | 0x0013 | Read | 0x0000 |
| reg_tx_control1 | 0x0014 | Read/Write | 0x0000 |
| reg_cmd_proc_start1 | 0x0015 | Read/Write | 0x0000 |
| reg_cmd_proc_length1 | 0x0016 | Read/Write | 0x0000 |
| reg_23 | 0x0017 |  | 0x0000 |
| reg_24 | 0x0018 |  | 0x0000 |
| reg_25 | 0x0019 |  | 0x0000 |
| reg_26 | 0x001A |  | 0x0000 |
| reg_27 | 0x001B |  | 0x0000 |
| reg_28 | 0x001C |  | 0x0000 |
| reg_29 | 0x001D |  | 0x0000 |
| reg_bus1_status | 0x001E |  | 0x0000 |
| reg_bus1_mask | 0x001F |  | 0x0000 |
| reg_status_rxed2 | 0x0020 | Read | 0x0000 |
| reg_mode_cmd_rxed2 | 0x0021 | Read | 0x0000 |
| reg_mode_data_rxed2 | 0x0022 | Read | 0x0000 |
| reg_rx_cmd2 | 0x0023 | Read | 0x0000 |
| reg_tx_control2 | 0x0024 | Read/Write | 0x0000 |
| reg_cmd_proc_start2 | 0x0025 | Read/Write | 0x0000 |
| reg_cmd_proc_length2 | 0x0026 | Read/Write | 0x0000 |
| reg_39 | 0x0027 |  | 0x0000 |
| reg_40 | 0x0028 |  | 0x0000 |
| reg_41 | 0x0029 |  | 0x0000 |
| reg_42 | 0x002A |  | 0x0000 |
| reg_43 | 0x002B |  | 0x0000 |
| reg_44 | 0x002C |  | 0x0000 |
| reg_45 | 0x002D |  | 0x0000 |
| reg_bus2_status | 0x002E |  | 0x0000 |
| reg_bus2_mask | 0x002F |  | 0x0000 |
| Timestamp3 MSB | 0x0030 | Read/Write | 0x0000 |
| Timestamp2 | 0x0031 | Read/Write | 0x0000 |
| Timestamp1 | 0x0032 | Read/Write | 0x0000 |
| Timestamp0 LSB | 0x0033 | Read/Write | 0x0000 |
| reg_rt_control | 0x0034 | Read/Write | 0x8000 |
| reg_err_inj_data | 0x0035 | Read/Write | 0x0000 |
| reg_gID | 0x0036 | Read/Write | 0x0000 |
| reg_fw_version | 0x0037 | Read | 0x0000 |
| reg_rt_addr | 0x0038 | Read | 0x0001 |

Table 2: Registers

### reg_status (0x0000)

| Read/Write | Default |
| --- | --- |
| Read | 0x0000 |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TxAvail1 | TxAvail2 | Tx1_Off | Tx2_Off | Inhibit T/F | DBC accepted | Synchronize | Initiate BIT |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

| TxAvail1 | 1 indicates that transmitter 1 is available, 0 indicates that the transmitter 1 is busy. |
| --- | --- |
| TxAvail2 | 1 indicates that transmitter 2 is available, 0 indicates that the transmitter 2 is busy. |
| Tx1_Off | 1 when a transmitter shutdown mode command is received for transmitter 1. This mode command would be received on bus 2. The bit will only be cleared when the clear transmitter shutdown mode command is received on bus 2. |
| Tx2_Off | 1 when a transmitter shutdown mode command is received for transmitter 2. This mode command would be received on bus 1. The bit will only be cleared when the clear transmitter shutdown mode command is received on bus 1. |
| Inhibit T/F | 1 indicates the “Inhibit T/F” mode command has be received. The bit will be cleared if the clear inhibit T/F mode command is received. The bit is a logical OR of the inhibit T/F flag of both MACs (bus 1 and bus 2). Thus both need to be cleared for the bit to be 0. |
| DBC_accepted | 1 indicates that the RT MAC accepted command to become bus controller. For this function to be active, the DBC Enable bit in the reg_rt_control also needs to be set to 1. The bit is a logical OR of both the MACs DBC Accepted bit. (NOTE: the bus controller functionality is not included in this core) |
| Synchronize | 1 indicates that a Synchronize mode command has been received on either bus 1 or bus 2. The bit is latched until cleared by the host. The host can clear this bit by writing a 1 to bit position 11 of the reg_clear_bits register. |
| InitiateBIT | 1 indicates that an Initiate Build in Selftest mode command has been received on either bus 1 or bus 2. The bit is latched until cleared by the host. The host can clear this bit by writing a 1 to bit position 11 of the reg_clear_bits register. |
| Reserved | Reserved for future use. Value is unspecified and may read back either 0 or 1 |

A bit that is set in the status register can generate an interrupt to the host CPU if its corresponding bit in the interrupt mask register (reg_intr_mask) is set. The user should ensure that bit 7 down to 0 are cleared in the mask register to avoid spurious interrupts from the lower byte of the status register

### reg_intr_mask (0x0001)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0x0000 |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TxAvail1 | TxAvail2 | Tx1_Off | Tx2_Off | Inhibit T/F | DBC accepted | Synchronize | Initiate BIT |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

| TxAvail1 | 1 enables the interrupt to indicate that transmitter 1 is available, 0 disables the interrupt |
| --- | --- |
| TxAvail2 | 1 enables the interrupt to indicate that transmitter 2 is available, 0 disables the interrupt |
| Tx1_Off | 1 enables the interrupt to indicate that transmitter 1 shutdown mode command has been received, 0 disables the interrupt |
| Tx2_Off | 1 enables the interrupt to indicate that transmitter 2 shutdown mode command has been received 0 disables the interrupt |
| Inhibit T/F | 1 enables the interrupt to indicate that either bus 1 or bus 2 received an inhibit T/F mode command, 0 disables the interrupt |
| DBC_accepted | 1 enables the interrupt to indicate that DBC mode command has been accepted by either bus 1 or bus2, 0 disables the interrupt |
| Synchronize | 1 enables the interrupt to indicate that a synchronize mode command was received on either but 1 or bus 2, 0 disables the interrupt |
| InitiateBIT | 1 enables the interrupt to indicate that an initiate BIT mode command has been received on either bus 1 or bus 2, 0 disables the interrupt |
| Reserved | Reserved for future use. Set to 0 to avoid spurious interrupts. |

### reg_clear_bits (0x0002)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0x0000 |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Reserved | Reserved | Reserved | Synchronize | InitiateBIT | Reserved | Reserved |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

| Synchronize | 1 commands the MAC to clear the latched bit. The command is sent to both MACs and the results is that the Synchronize bit in the status register (reg_status.9) is cleared. This bit clears automatically and will be read back as 0. |
| --- | --- |
| Initiate BIT | 1 commands the MAC to clear the latched bit. The command is sent to both MACs and the results is that the InitiateBIT bit in the status register (reg_status.8) is cleared. This bit clears automatically and will be read back as 0. |
| Reserved | Reserved for future use. Set to 0 to avoid spurious interrupts. |

### reg_tmr_1us (0x0003)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0x0063 |

This register controls the timeout of the 1us timer. This 1us timer is used to enable the safety timer (SAF). The value in SAF timer is thus given in increments of the 1us timer value. Typically this timer value should not be modified, but during simulations it is convenient to speed up some timer actions for testing purposes.

The default (0x0063) is the correct value for operating from a 100MHz internal clock. The value to set is calculated by:

reg_tmr_1us=1us*100Mhz-1

### reg_tmr_GAP (0x0004)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0x018F |

This register controls the intermessage gap time. The specification calls for a minimum intermessage gap of 4us, but also calls for a response of 12us. The GAP timer must thus be set to a value between these limits. Typically this timer value should not be modified, but during simulations it is convenient to speed up some timer actions for testing purposes.

The default (0x018F) is the correct value for operating from a 100MHz internal clock. The value to set is calculated by:

reg_tmr_GAP=4us*100Mhz-1

### reg_tmr_NRP (0x0005)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0x0577 |

This register controls the no response time. The specification calls for a no response time of 14us. Typically this timer value should not be modified, but during simulations it is convenient to speed up some timer actions for testing purposes.

The default (0x0577) is the correct value for operating from a 100MHz internal clock. The value to set is calculated by:

reg_tmr_NRP=14us*100Mhz-1

### reg_tmr_REP (0x0006)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0x018F |

This register controls the response time to a valid command. The specification calls for a response time between 4us and 12us. The REP timer must thus be set to a value between these limits. Typically this timer value should not be modified, but during simulations it is convenient to speed up some timer actions for testing purposes.

The default (0x018F) is the correct value for operating from a 100MHz internal clock. The value to set is calculated by:

reg_tmr_REP=4us*100Mhz-1

### reg_tmr_SAF (0x0007)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0x031F |

This register controls the safety timer. The specification calls for a terminal fail safe of 800us. Typically this timer value should not be modified, but during simulations it is convenient to speed up some timer actions for testing purposes.

The default (0x031F) is the correct value for operating from a 100MHz internal clock when the reg_tmr_1us is set to the correct value for 1us. The SAF timer increments in 1us intervals.  The value to set is calculated by:

reg_tmr_SAF=800us1us-1

### reg_legal_subaddr1 (0x0008)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0xFFFF |

The reg_legal_subaddr1 and reg_legal_subaddr2 are combined to form a 32bit LegalSubAddr value with the reg_legal_subaddr1 being the least significant bits.

Each bit in this LegalSubAddr relates to the sub address sent by the bus controller. The remote terminal will only respond to sub addresses where the bit in this LegalSubAddr register is set to 1.

The default value (0xFFFF) allows sub addresses 0 to 15, however bit 0 is ignored since it has the special function of indicating mode commands.

### reg_legal_subaddr2 (0x0009)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0xFFFF |

The reg_legal_subaddr1 and reg_legal_subaddr2 are combined to form a 32bit LegalSubAddr value with the reg_legal_subaddr1 being the least significant bits.

Each bit in this LegalSubAddr relates to the sub address sent by the bus controller. The remote terminal will only respond to sub addresses where the bit in this LegalSubAddr register is set to 1.

The default value (0xFFFF) allows sub addresses 16 to 32, however bit 31 is ignored since it has the special function of indicating mode commands.

### reg_legal_rx_mode1 (0x000A)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0x0000 |

The reg_legal_rx_mode1 and reg_legal_rx_mode2 are combined to form a 32bit LegalModeRx value with the reg_legal_rx_mode1 being the least significant bits.

Each bit in this LegalModeRx relates to the mode command sent by the bus controller where the controller expects a data word reply. The remote terminal will only respond to mode commands where the bit in this LegalModeRx register is set to 1.

The default value (0x0000) is as per TABLE I in [1].

### reg_legal_rx_mode2 (0x000B)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0x0032 |

The reg_legal_rx_mode1 and reg_legal_rx_mode2 are combined to form a 32bit LegalModeRx value with the reg_legal_tx_mode1 being the least significant bits.

Each bit in this LegalModeRx relates to the mode command sent by the bus controller where the controller expects a data word reply. The remote terminal will only respond to mode commands where the bit in this LegalModeRx register is set to 1.

The default value (0x0032) is as per TABLE I in [1], allowing the following mode commands:

- Synchronize

- Selected Transmitter shutdown

- Override Selected Transmitter shutdown

### reg_legal_tx_mode1 (0x000C)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0x01FF |

The reg_legal_tx_mode1 and reg_legal_tx_mode2 are combined to form a 32bit LegalModeTx value with the reg_legal_rx_mode1 being the least significant bits.

Each bit in this LegalModeTx relates to the mode command sent by the bus controller. The remote terminal will only respond to mode commands where the bit in this LegalModeTx register is set to 1.

The default value (0x01FF) is as per TABLE I in [1], allowing the following mode commands:

- Dynamic Bus control

- Synchronize

- Transmit Status Word

- Initiate Self Test

- Transmitter Shutdown

- Override Transmitter Shutdown

- Inhibit Terminal Flag Bit

- Override Inhibit Terminal Flag Bit

- Reset Remote Terminal

### reg_legal_tx_mode2 (0x000D)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0x000D |

The reg_legal_tx_mode1 and reg_legal_tx_mode2 are combined to form a 32bit LegalModeTx value with the reg_legal_rx_mode1 being the least significant bits.

Each bit in this LegalModeTx relates to the mode command sent by the bus controller. The remote terminal will only respond to mode commands where the bit in this LegalModeTx register is set to 1.

The default value (0x000D) is as per TABLE I in [1], allowing the following mode commands:

- Transmit Vector Word

- Transmit Last Command

- Transmit BIT word

### reg_legal_brdcst_mode1 (0x000E)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0x01FA |

The reg_legal_brdcst_mode1 and reg_legal_brdcst_mode2 are combined to form a 32bit LegalModeBrdcst value with the reg_legal_brdcst_mode1 being the least significant bits.

Each bit in this LegalModeBrdcst relates to the mode command sent by the bus controller. The remote terminal will only respond to mode commands where the bit in this LegalModeBrdcst register is set to 1 when the station ID is the broadcast station ID.

The default value (0x01FA) is as per TABLE I in [1], allowing the following broadcast mode commands:

- Synchronize

- Initiate Self Test

- Transmitter Shutdown

- Override Transmitter Shutdown

- Inhibit Terminal Flag Bit

- Override Inhibit Terminal Flag Bit

- Reset Remote Terminal

### reg_legal_brdcst_mode2 (0x000F)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0x0032 |

The reg_legal_brdcst_mode1 and reg_legal_brdcst_mode2 are combined to form a 32bit LegalModeBrdcst value with the reg_legal_brdcst_mode1 being the least significant bits.

Each bit in this LegalModeBrdcst relates to the mode command sent by the bus controller. The remote terminal will only respond to mode commands where the bit in this LegalModeBrdcst register is set to 1 when the station ID is the broadcast station ID.

The default value (0x0032) is as per TABLE I in [1], allowing the following broadcast mode commands:

- Synchronize

- Selected Transmitter shutdown

- Override Selected Transmitter shutdown

### reg_status_rxed1 (0x0010) and reg_status_rxed2 (0x0020)

These two registers are identical in function and each relates to the status word received on its bus. The ParityOK bit (bit 7) is replaced with the currently calculated parity of the address wires.

| Read/Write | Default |
| --- | --- |
| Read | N/A |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RtAddr4 | RtAddr3 | RTAddr2 | RtAddr1 | RtAddr0 | MessageError | Instrument | ServiceReq |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RtAddrError | Reserved | Reserved | Brdcst Rxed | Busy | SubSysFlag | DBC Acpt | TerminalFlag |

| RtAddr(4..0) | Contains the address of the transmitting node |
| --- | --- |
| Message Error | Indicates that there was an error in the preceding data words. |
| Instrument | Reserved for instrumentation bit and should always be 0 |
| Service Request | When received by a bus controller, this bit indicates that the RT or associated subsystem requires service |
| RtAddrError | Indicates an error on local address lines |
| Brdcst_Rxed | A logic 1 indicates that the preceding valid command word was a broadcast command and a logic 0 shows it was not a broadcast command. |
| Busy | A logic 1 indicates that the RT is still busy and not ready to respond to new commands. |
| SubSysFlag | This bit shall flag a subsystem fault condition, and alert the bus controller to potentially invalid data. |
| DBC_Acpt | A logic one shall indicate acceptance of control, and a logic zero shall indicate rejection of control. |
| Terminal Flag | This bit shall flag a RT fault condition |
| Reserved | Reserved for future use. |

### reg_mode_cmd_rxed1 (0x0011) and reg_mode_cmd_rxed2 (0x0021)

These two registers contain the exact mode command received on each bus.

| Read/Write | Default |
| --- | --- |
| Read | N/A |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RtAddr4 | RtAddr3 | RTAddr2 | RtAddr1 | RtAddr0 | Transmit/Receive | Mode4 | Mode3 |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mode2 | Mode1 | Mode0 | ModeCode4 | ModeCode3 | ModeCode2 | ModeCode1 | ModeCode0 |

| RtAddr(4..0) | Contains the RT address |
| --- | --- |
| Transmit/Receive | Indicates transmit or receive command |
| Mode(4..0) | Mode command bits, either “00000” or “11111” |
| ModeCode(4..0) | Mode command Code. Some mode codes indicates that a data word shall follow. The data word will be placed in reg_mode_data_rxed1/2 |
| Reserved | Reserved for future use. |

### reg_mode_data_rxed1 (0x0012) and reg_mode_data_rxed2 (0x0022)

The contents of this register depends on the mode command code. These registers contain the received mode command data word verbatim.

| Read/Write | Default |
| --- | --- |
| Read | N/A |

### reg_rx_cmd1 (0x0013) and reg_rx_cmd2 (0x0023)

These registers contain a latched version of the last command received and should be used in conjunction with the NewCmdReceived interrupt.

| Read/Write | Default |
| --- | --- |
| Read | N/A |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Reserved | Reserved | OtherTxOff | Inhibit T/F | DBC | Synchronize | InitiateBIT |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

| OtherTxOff | State of other transmitter. If received on Bus 1, indicates the status of the Bus2 transmitter. If 1, turn the transmitter off, if 0 turn the transmitter on. |
| --- | --- |
| Inhibit T/F | This command shall cause the RT to control T/F bit. When 1, the T/F bit is inhibited, when 0, it is not inhibited. |
| DBC | Dynamic Bus control command received |
| Synchronize | This command shall cause the RT to synchronize |
| Initiate BIT | This command shall be used to initiate self test within the RT |
| Reserved | Reserved for future use. |

### reg_bus1_status (0x001E) and reg_bus2_status (0x002E)

| Read/Write | Default |
| --- | --- |
| Read | N/A |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ModeCmd | DataReq | DataReceived | StatusRxedFlag | SAF | REP | NRP | GAP |

| ModeCmd | Indicates that a ModeCmd word has been received |
| --- | --- |
| DataReq | Indicates that a data word is requested. The host should ensure that the expected data word is updata. |
| DataReceived | Indicates that the RT received data. The host should read the data from the Rx RAM. |
| StatusRxedFlag | Indicates that a status word has been received |
| SAF | Indicates that the SAF safety timer has expired |
| REP | Indicates that the REP response timer has expired |
| NRP | Indicates that the NRP No response timer has expired |
| GAP | Indicates that the GAP intermessage gap timer has expired |
| Reserved | Reserved for future use |

A bit that is set in the status register can generate an interrupt to the host CPU if its corresponding bit in the interrupt mask register (reg_bus#_mask) is set. The user should ensure that bit 15 down to 8 are cleared in the mask register to avoid spurious interrupts from the lower byte of the status register

### reg_bus1_mask (0x001F) and reg_bus2_mask (0x002F)

| Read/Write | Default |
| --- | --- |
| Read/Write | N/A |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ModeCmd | DataReq | DataReceived | StatusRxedFlag | SAF | REP | NRP | GAP |

| ModeCmd | 1 enables interrupt generation for received mode command |
| --- | --- |
| DataReq | 1 enables interrupt generation for data requested mode command |
| DataReceived | 1 enabled interrupt generation for received data |
| StatusRxedFlag | 1 enables interrupt generation for the receipt of a status word |
| SAF | 1 enables interrupt generation for the expiration of the SAF timer |
| REP | 1 enables interrupt generation for the expiration of the REP timer |
| NRP | 1 enables interrupt generation for the expiration of the NRP timer |
| GAP | 1 enables interrupt generation for the expiration of the GAP timer |
| Reserved | Reserved for future use. Set to 0 to avoid spurious interrupts |

### Timestamp (0x0030 – 0x0033)

These four registers contain the core 64-bit system time counter. After reset of the core, this counter increments on each clock cycle. When the maximum value is reached, the counter wraps around to zero again.

The address 0x0030 contains the most significant word and the address 0x0033 the least significant word.

### reg_rt_control (0x0034)

| Read/Write | Default |
| --- | --- |
| Read/Write | N/A |

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BusBusy | BrdcstEn | DBC_En | Reserved | Reserved | Reserved | Reserved | Reserved |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

| BusBusy | This bit is inserted into the status word of this RT. The Host needs to clear this bit after completing configuration. |
| --- | --- |
| BrdcstEn | This bit control the validity of broadcast mode |
| DBC_En | This bit will enable the dynamic bus control mode. Not implement. |
| Reserved | Reserved for future use. Set to 0 to avoid spurious interrupts |

### reg_gID (0x0036)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0xBEEF |

This register can be used by the host to identify a specific core in case of multiple core instantiations. The host is also free to use this register as a general scratch register. The contents are not used within the core.

### reg_fw_version (0x0037)

| Read/Write | Default |
| --- | --- |
| Read/Write | 0x0100 |

This register indicates the current version of the firmware for the core. The MSB nibble indicates the major version and the LSB nibble the minor version.

### reg_rt_addr (0x0038)

| Read/Write | Default |
| --- | --- |
| Read/Write | N/A |

This register contains the logical states of the actual RT address lines and parity lines as connected to the core.

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | Reserved | RtAddrParity | RtAddr5 | RtAddr3 | RtAddr2 | RtAddr1 | RtAddr0 |

## Transmit RAM

| Start | End | Size | Function |
| --- | --- | --- | --- |
| 0x0400 | 0x041F | 32 | Reserved |
| 0x0420 | 0x043F | 32 | SubAddr1 |
| 0x0440 | 0x045F | 32 | SubAddr2 |
| 0x0460 | 0x047F | 32 | SubAddr3 |
| 0x0480 | 0x049F | 32 | SubAddr4 |
| 0x04A0 | 0x04BF | 32 | SubAddr5 |
| 0x04C0 | 0x04DF | 32 | SubAddr6 |
| 0x04E0 | 0x04FF | 32 | SubAddr7 |
| 0x0500 | 0x051F | 32 | SubAddr8 |
| 0x0520 | 0x053F | 32 | SubAddr9 |
| 0x0540 | 0x055F | 32 | SubAddr10 |
| 0x0560 | 0x057F | 32 | SubAddr11 |
| 0x0580 | 0x059F | 32 | SubAddr12 |
| 0x05A0 | 0x05BF | 32 | SubAddr13 |
| 0x05C0 | 0x05DF | 32 | SubAddr14 |
| 0x05E0 | 0x05FF | 32 | SubAddr15 |
| 0x0600 | 0x061F | 32 | SubAddr16 |
| 0x0620 | 0x063F | 32 | SubAddr17 |
| 0x0640 | 0x065F | 32 | SubAddr18 |
| 0x0660 | 0x067F | 32 | SubAddr19 |
| 0x0680 | 0x069F | 32 | SubAddr20 |
| 0x06A0 | 0x06BF | 32 | SubAddr21 |
| 0x06C0 | 0x06DF | 32 | SubAddr22 |
| 0x06E0 | 0x06FF | 32 | SubAddr23 |
| 0x0700 | 0x071F | 32 | SubAddr24 |
| 0x0720 | 0x073F | 32 | SubAddr25 |
| 0x0740 | 0x075F | 32 | SubAddr26 |
| 0x0760 | 0x077F | 32 | SubAddr27 |
| 0x0780 | 0x079F | 32 | SubAddr28 |
| 0x07A0 | 0x07BF | 32 | SubAddr29 |
| 0x07C0 | 0x07DF | 32 | SubAddr30 |
| 0x07E0 | 0x07FF | 32 | MODE Data |

Table 3: Transmit RAM Memory Map

The Transmit RAM is divided into 32 blocks of 32 words each. Each block is dedicated to a specific sub-address. The host shall ensure that the data within these blocks are correctly configured. The core will read the data from this location as needed.

Tx RAM address 0x0400 is reserved since sub-address 0 is sued for command words.

Tx RAM address 0x7C0 contains the data of the mode commands when the mode command requests a data word.

## Receive RAM

| Start | End | Size | Function |
| --- | --- | --- | --- |
| 0x0800 | 0x081F | 32 | Not used |
| 0x0820 | 0x083F | 32 | SubAddr1 |
| 0x0840 | 0x085F | 32 | SubAddr2 |
| 0x0860 | 0x087F | 32 | SubAddr3 |
| 0x0880 | 0x089F | 32 | SubAddr4 |
| 0x08A0 | 0x08BF | 32 | SubAddr5 |
| 0x08C0 | 0x08DF | 32 | SubAddr6 |
| 0x08E0 | 0x08FF | 32 | SubAddr7 |
| 0x0900 | 0x091F | 32 | SubAddr8 |
| 0x0920 | 0x093F | 32 | SubAddr9 |
| 0x0940 | 0x095F | 32 | SubAddr10 |
| 0x0960 | 0x097F | 32 | SubAddr11 |
| 0x0980 | 0x099F | 32 | SubAddr12 |
| 0x09A0 | 0x09BF | 32 | SubAddr13 |
| 0x09C0 | 0x09DF | 32 | SubAddr14 |
| 0x09E0 | 0x09FF | 32 | SubAddr15 |
| 0x0A00 | 0x0A1F | 32 | SubAddr16 |
| 0x0A20 | 0x0A3F | 32 | SubAddr17 |
| 0x0A40 | 0x0A5F | 32 | SubAddr18 |
| 0x0A60 | 0x0A7F | 32 | SubAddr19 |
| 0x0A80 | 0x0A9F | 32 | SubAddr20 |
| 0x0AA0 | 0x0ABF | 32 | SubAddr21 |
| 0x0AC0 | 0x0ADF | 32 | SubAddr22 |
| 0x0AE0 | 0x0AFF | 32 | SubAddr23 |
| 0x0B00 | 0x0B1F | 32 | SubAddr24 |
| 0x0B20 | 0x0B3F | 32 | SubAddr25 |
| 0x0B40 | 0x0B5F | 32 | SubAddr26 |
| 0x0B60 | 0x0B7F | 32 | SubAddr27 |
| 0x0B80 | 0x0B9F | 32 | SubAddr28 |
| 0x0BA0 | 0x0BBF | 32 | SubAddr29 |
| 0x0BC0 | 0x0BDF | 32 | SubAddr30 |
| 0x0BE0 | 0x0BFF | 32 | MODE Data |

Table 4: Receive RAM Memory Map

The Receive RAM is divided into 32 blocks of 32 words each. Each block is dedicated to a specific sub-address. The host shall read the data from this location when indicated by data received interrupt.

Rx RAM address 0x0800 is reserved since sub-address 0 is used for command words.

Rx RAM address 0xBE0 contains the data of the mode commands when the mode command contains a data word.

# Remote Terminal Configuration

After power on reset (nReset) signal is released high, the core registers are configured to default values. The core will perform a parity check on the connected RT address lines and parity line. If a parity error is present, the core will remain in reset, but not the CPU interface. The host can check for this condition by investigating reg_rt_addr to see the values of the connected signals as well as reg_status_rxed#.RtAddrError.

The host needs to configure the transmit RAM with correct message data. Once this is complete, the host needs to clear the reg_rt_control.BusBusy bit.

The host also needs to configure the interrupts as required.

# Testbench

The Testbench is built with 2 RTs and one BC. Each RT and BC has a commands.txt file that controls it CPU address side. The bus side are connected together.

The testbench is designed to be able to perform each of the 65535 test cases. The text command files for these tests are generated by a QT application. These command.txt files are loaded by each of the 3 command processor block in the testbench. The command processor block also contain generic input and output signals. These signal can be used to check for events, i.e. connecting to the IRQ output of the CPU interface on the core.

In this testbench, these discrete are also used to synchronise the 3 command processors before and after a test. The BC initiates a new test by setting IO(15) high. The RTs indicate completion of a test by setting their IO(15) high. Each of their IO(15) lines are routed to the input on the BC at IO(11) and IO(12) respectively for RT1 and RT2. The BC will check these lines, and if both are high proceed by setting BCIO(15) high again. The RTs would then clear their output signal. Once the BC detect that both RT signals are low, it clear it output signal and actually start the first test command.

The sequence below is in BC file before every new test case. At the end of a test for an RT, the RT just need to set is “Test Complete “ signal high.

WI 11 1 0  waiting for the RT1 to set “complete” signal high

WI 12 1 0  waiting for the RT2 to set “complete” signal high

WO 15 1  now the BC sets it “Start Test” signal high

WI 11 0 0  waiting without timeout for RT1 to clear its “complete” signal

WI 12 0 0  waiting without timeout for RT2 to clear its “complete” signal

WO 15 0  BC clears it “Start Test” signal, the actual test case will now commence