MIL-STD-1553 CORE FIRMWARE TEST PLAN

??

ISSUE 1

STATEMENT OF CONFIDENTIALITY

All information in this document is Barij Dynamics LLC (EBD-AT), trading as Al Tariq, PRIVILEGED and for INTERNAL USE ONLY. The information in this document contains trade secrets, confidential and proprietary information of Barij Dynamics, the disclosure of which would provide substantial benefit to competitors.  As a result, this document should not be disclosed, used or duplicated – in whole or in part without the explicit authorization of Barij Dynamics. The information is intended solely for the use of authorized recipients. If you are not the intended recipient, you are prohibited from reading, using, disseminating, distributing and/or copying this document. In accessing the document, the reader warrants that the information contained herein will be treated as internal use only and that no content of any nature will be shared with any other external party or unauthorized recipient.

TITLE PAGE

| TITLE | : | MIL-STD-1553 CORE FIRMWARE TEST PLAN |  |
| --- | --- | --- | --- |
| DOCUMENT NUMBER | : | ?? | ISSUE 1 |
| CLASSIFICATION | : | RESTRICTED |  |
| SYNOPSIS | : | Test plan for the MIL-STD-1553 Firmware Core. |  |
| PREPARED BY | : | J Smith |  |
| APPROVED BY | : | Refer to approval page. |  |
| DATE | : | JUNE 2025 |  |

APPROVAL PAGE

This document is approved for release to users by:

??	??	:	_________________________

??	??	:	_________________________

??	??	: 	_________________________

CoNTENTS

Page

ABBREVIATIONS	5

1.	Scope	6

1.1	Identification	6

1.2	System overview	6

1.3	Document overview	7

2.	Related documents	8

2.1	Reference documents	8

2.2	Applicable documents	8

3.	Version description	9

3.1	Inventory of materials released	9

3.2	Inventory of software contents	9

3.3	Changes installed	9

3.4	Adaptation data	11

3.5	Related documents	11

3.6	Installation instructions	11

3.7	Possible problems and known errors	15

4.	Notes	16

FIGURES

Figure 1: Read Meta Data Prior to download	14

Figure 2: Read Meta Data After download	15

TABLES

Table 1: Executable zip File	9

Table 2: EXECUTABLE BINARY	9

Table 3: Source zip file	9

Table 5: Prerequisites for creating binary file	11

Table 6: Application Software for loading a binary file to hardware	13

Table 7: LoaderApplication.xml Parameters	13

ABBREVIATIONS

| BC | Bus Controller |
| --- | --- |
| RT | Remote Terminal |
| BKS | Bomb Kit System |
| CRC | Cyclic Redundancy Check |
| CSCI | Computer Software Configuration Item |
| FW | Firmware |
| LAN | Local Area Network |
| PC | Personal Computer |
| PCM | Pulse-code Modulation |
| PDI | Product Data Index |
| SW | Software |
| Telpack | Telemetry Pack |
| URL | Uniform Resource Locator |
| USB | Universal Serial Bus |
| VDD | Version Description Document |

# Scope

## Identification

This document is the formal Test Plan for the MIL-STD-1553C Core developed for use by Al-Tariq, hereafter referred to as the 1553 Core. It identifies the verification strategy, methods, and responsibilities associated with validating the conformance, functionality, and robustness of the core as per the standards referenced in 2.1.

## System overview

This Test Plan describes the verification and validation activities required to demonstrate that the 1553 Core complies with applicable electrical, functional, and protocol-level requirements defined in MIL-STD-1553B and the related AS411x series. The scope includes:

- Manchester Encoder and Decoder:

  - Generation and interpretation of Manchester II (bi-phase-level) encoded signals.

  - Detection of protocol violations and error conditions, including parity, sync, and transition errors.

- Protocol Layer:

  - Correct generation and interpretation of command, status, and data words.

  - Support for RT-to-RT, BC-to-RT, and RT-to-BC communication.

  - Compliance with sub addressing, mode codes, and parity bit handling.

- Bus Interface and Timing:

  - Bit, word, and inter-message timing validation per 1553B specifications.

  - Gap timers, response time compliance, and message framing.

- Fault Injection and Error Handling:

  - System behaviour under conditions including parity error, jitter, missing transitions, inverted polarity, sync corruption, and bus contention.

- Compliance Testing:

  - Execution of tests defined in AS4111 and AS4113 for both component-level and terminal-level functionality.

The test plan includes simulation-based validation using OSVVM, automated self-checking testbenches, and waveform-based verification techniques. It is structured to allow modular test execution of each subsystem (encoder, decoder, protocol handler) and supports reuse for integration testing and hardware verification on FPGA.

The overall objective is to establish a high-confidence verification baseline that ensures the 1553 Core can be integrated into mission- or safety-critical systems with predictable behavior under nominal and fault conditions.

# Related documents

## Reference documents

- MIL-STD-1553C	:	Digital Time Division Command/Response Multiplex Data Bus. United States Department of Defense, February 2018.

## Applicable documents

- AS4111 Rev A	:	Validation Test Plan for the Digital Time Division Command/Response Multiplex Data Bus Remote Terminals

- AS4113 Rev A	:	Validation Test Plan for the Digital Time Division Command/Response Multiplex Data Bus Bus Controllers

# Firmware Test Environment

## Tools

The following tools are used for the 1553 core and test bench implementation:

- Siemens HDL Designer 2024.1  Design capture

- QuestaSim-64 2024.1  Mixed language simulation

- OSVVM 2025.04  Automated test benches

## Inventory of Firmware/Software contents

This section will be updated towards the end of the testing process to reflect the firmware tested as well as the test benches created for this purpose

# Test identification

## General information

### Test levels

### Test classes

### General test conditions

### Test progression

### Data recording, reduction, and analysis

## Planned tests

### Input Synchroniser

### General Timer

### Avalon CPU Interface

### Active Bus Selection

### Manchester-II Encoder

### Manchester-II Decoder

### Protocol Tests

These tests assess the Bus Controller’s/Remote Terminal’s adherence to the MIL-STD-1553C protocol specifications.  All the test in sthis section are performed using the test setup in Figure 1.

Add test setup figure here

Figure 1: Test setup for protocol testing

#### Command Vocabulary Verification

This test set verifies that the BC can transmit every command word it is designed for:

- Each unique command word shall be transmitted.

- Receive command words shall be followed by the correct number of data words.

Table 1: Command Vocabulary Verification Procedures

| Test ID | Objective | Message Type | Verification Points |
| --- | --- | --- | --- |
| BC-001 | BC sends Receive command word with 1 to 32 data words. This sequence is then repeated for all RT addresses except the broadcast “11111”. Total messages = 32*31= 992 | BC to RT RECEIVE CMD | RT checks that the command word and correct number and contents of data words are received. BC checks clear status (CS) reply from RT |
| BC-002 | Verify BC can request multiple data words from RT | RT to BC | Correct data words received in sequence; word count and timing validated |

| Test ID | Objective | Message Type | Verification Points |
| --- | --- | --- | --- |
| BC-001 | Verify BC can request 1 data word from RT | RT to BC | BC sends command; RT returns 1 data word + status; BC decodes and validates |
| BC-002 | Verify BC can request multiple data words from RT | RT to BC | Correct data words received in sequence; word count and timing validated |
| BC-003 | Verify BC can transmit 1 data word to RT | BC to RT | BC sends command + 1 data word; RT receives; no response expected |
| BC-004 | Verify BC can transmit multiple data words to RT | BC to RT | BC sends N data words; RT acknowledges via status; no data returned |
| BC-005 | Verify BC can conduct RT-to-RT transfer | RT to RT | BC mediates: one RT transmits, one RT receives; both status words received |
| BC-006 | Send mode code without data | Mode Code | RT receives mode code; responds with correct status |
| BC-007 | Send mode code with one data word | Mode Code | RT sends/receives single word depending on mode code; status word returned |
| BC-008 | Broadcast transmission of data | Broadcast | All RTs accept data silently; no responses expected |
| BC-009 | Broadcast mode code | Broadcast | RTs update internal state; no response; verify no unexpected transmission |
| BC-010 | Send command with word count 0 | Edge Case | BC/RT handle 0-word command appropriately (mode code or reserved behavior) |
| BC-011 | Send command with maximum word count (32) | Edge Case | BC sends 32 data words; receiver handles and validates all words |
| BC-012 | Send command to non-existent RT | Error Handling | No response; BC detects timeout or marks error |
| BC-013 | No response from RT | Error Handling | BC detects timeout and handles according to protocol |
| BC-014 | RT responds too early | Timing Error | BC detects early response and flags timing violation |
| BC-015 | RT responds too late | Timing Error | BC detects delayed response and flags timing violation |
| BC-016 | RT responds with incorrect status | Error Handling | BC detects mismatch in expected status word bits |
| BC-017 | RT responds with parity error | Error Handling | BC detects parity fault in response word |
| BC-018 | RT sends unexpected word count | Error Handling | BC detects more/fewer data words than commanded |
| BC-019 | Verify inter-word and inter-message gap timing | Protocol Timing | All gaps and spacing validated against 1553 timing rules |
| BC-020 | Sync pattern is corrupted mid-transfer | Fault Injection | BC fails to decode word and flags sync error |

##### BC-001 – Verify BC can request 1 data word from RT

| Test ID | BC-001 |
| --- | --- |
| Objective | Verify BC can request 1 data word from RT |
| Message Type | RT to BC |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-002 – Verify BC can request multiple data words from RT

| Test ID | BC-002 |
| --- | --- |
| Objective | Verify BC can request multiple data words from RT |
| Message Type | RT to BC |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-003 – Verify BC can transmit 1 data word to RT

| Test ID | BC-003 |
| --- | --- |
| Objective | Verify BC can transmit 1 data word to RT |
| Message Type | BC to RT |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-004 – Verify BC can transmit multiple data words to RT

| Test ID | BC-004 |
| --- | --- |
| Objective | Verify BC can transmit multiple data words to RT |
| Message Type | BC to RT |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-005 – Verify BC can conduct RT-to-RT transfer

| Test ID | BC-005 |
| --- | --- |
| Objective | Verify BC can conduct RT-to-RT transfer |
| Message Type | RT to RT |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-006 – Send mode code without data

| Test ID | BC-006 |
| --- | --- |
| Objective | Send mode code without data |
| Message Type | Mode Code |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-007 – Send mode code with one data word

| Test ID | BC-007 |
| --- | --- |
| Objective | Send mode code with one data word |
| Message Type | Mode Code |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-008 – Broadcast transmission of data

| Test ID | BC-008 |
| --- | --- |
| Objective | Broadcast transmission of data |
| Message Type | Broadcast |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-009 – Broadcast mode code

| Test ID | BC-009 |
| --- | --- |
| Objective | Broadcast mode code |
| Message Type | Broadcast |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-010 – Send command with word count 0

| Test ID | BC-010 |
| --- | --- |
| Objective | Send command with word count 0 |
| Message Type | Edge Case |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-011 – Send command with maximum word count (32)

| Test ID | BC-011 |
| --- | --- |
| Objective | Send command with maximum word count (32) |
| Message Type | Edge Case |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-012 – Send command to non-existent RT

| Test ID | BC-012 |
| --- | --- |
| Objective | Send command to non-existent RT |
| Message Type | Error Handling |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-013 – No response from RT

| Test ID | BC-013 |
| --- | --- |
| Objective | No response from RT |
| Message Type | Error Handling |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-014 – RT responds too early

| Test ID | BC-014 |
| --- | --- |
| Objective | RT responds too early |
| Message Type | Timing Error |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-015 – RT responds too late

| Test ID | BC-015 |
| --- | --- |
| Objective | RT responds too late |
| Message Type | Timing Error |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-016 – RT responds with incorrect status

| Test ID | BC-016 |
| --- | --- |
| Objective | RT responds with incorrect status |
| Message Type | Error Handling |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-017 – RT responds with parity error

| Test ID | BC-017 |
| --- | --- |
| Objective | RT responds with parity error |
| Message Type | Error Handling |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-018 – RT sends unexpected word count

| Test ID | BC-018 |
| --- | --- |
| Objective | RT sends unexpected word count |
| Message Type | Error Handling |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-019 – Verify inter-word and inter-message gap timing

| Test ID | BC-019 |
| --- | --- |
| Objective | Verify inter-word and inter-message gap timing |
| Message Type | Protocol Timing |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

##### BC-020 – Sync pattern is corrupted mid-transfer

| Test ID | BC-020 |
| --- | --- |
| Objective | Sync pattern is corrupted mid-transfer |
| Message Type | Fault Injection |
| Preconditions | RT and BC cores initialized and clocked; simulation testbench is active. |
| Inputs |  |
| Expected Outputs |  |
| Pass Criteria | System behaves as described in "Expected Outputs" with no protocol violations or simulation errors |
| Error Injection | None unless otherwise stated |
| Verification Method | OSVVM-based automated testbench and waveform trace |

#### Response Time Verification

##### Measuring the time taken by the BC to respond to various conditions, ensuring compliance with specified timing requirements.

#### Error Handling

#### Testing the BC’s ability to detect and respond to errors such as parity errors, invalid commands, and incorrect status words.

#### Mode Code Processing

#### Validating the BC’s handling of various mode codes, including those for synchronization and status reporting.

#### Data Word Transmission and Reception

#### Verifying the correct handling of data words in both transmission and reception scenarios.