Transfer Alignment Parameters:

| TRANSFER ALIGNMENT PARAMETERS | NOTES |
| --- | --- |
| Validity Word |  |
| Roll |  |
| Pitch |  |
| Heading |  |
| Roll Rate |  |
| Pitch Rate |  |
| Yaw Rate |  |
| Velocity North |  |
| Velocity East |  |
| Velocity Down |  |
| Latitude |  |
| Longitude |  |
| Altitude |  |
| Lever Arm X |  |
| Lever Arm Y |  |
| Lever Arm Z |  |
| Latency 1 | for attitudes |
| Latency 2 | for angular rates |
| Latency 3 | for velocities |
| Latency 4 | for position |
| True Airspeed | optional |
| Calibrated Airspeed | optional |
| Static Pressure | optional |
| Angle of Attack | optional |
| Angle of Sideslip | optional |

Continuous-BIT:

| PAYLOAD ID 17 |  |
| --- | --- |
| Weapon C-BIT Status Word | Weapon C-BIT Status Word |
| BREAKOUT | bit 0: MCP BIT bit 1: IMU BIT bit 2: GNSS BIT bit 3: Pitot Controller BIT bit 4: Magnetometer BIT bit 5: ITU BIT bit 6: Servo BIT bit 7: Wireless Link BIT bit 8: Seeker BIT bit 9: Battery BIT bit 10: RTAC BIT bit 11: (reserved) bit 12: IMU Communication bit 13: GNSS Communication bit 14: Pitot Communication bit 15: Magnetometer Communication bit 16: ITU Communication bit 17: Servo Communication bit 18: Wireless Link Communication bit 19: Seeker Communication bit 20: ArcNet Recon bit 21: RTAC Communication bit 22: (reserved) bit 23: Tail Fin BIT bit 24: Page Fault bit 25: Floating Point bit 26: Memory Align bit 27: Stack Overflow bit 28: (reserved) bit 29: (reserved) bit 30: (reserved) bit 31: Weapon BIT Summary Bits 4 and 15 are never critical failures. Bits 7 and 18 are only critical failures for the IIR variant. Bits 8 and 19 are only critical failures for SAL or IIR mission types. Bits 20, 24, 25, 26 and 27 indicates software failures If any critical bit is set, the Weapon BIT Summary (bit 31) shall be set. |

Mission Parameters currently in Status Message

IIR Image CRC

Estimated Dive Angle

Estimated Attack Angle

Estimated Impact Speed

Target Latitude

Target Longitude

Target Altitude

Laser Period

Fuzing Mode

Arming Delay

Impact Delay

Fuzing Distance

IIR Image Index

LAR Parameters:

For each Configuration:

- Long / Standard Range

- Mk-81 / Mk-82

Dynamic Parameters:

- Altitude (5 000 ft to 40 000 ft)

- Mach Number (Mach 0.7 to Mach 0.9)

- Relative Attack Angle (-180° to +180°) – only above 15 000 ft

Static Parameters:

- Dive Angle (30° to 90° – increment 15°) – only above 15 000 ft

- Impact Speed (Normal / High) – only above 15 000 ft

- Target Type (Static / Moving)

Wind correction shall be done by real-time adjustment.